Instruction Manual for Software Updating

※Caution: To have power available for updating software properly, you must recharge the transmitter battery if its capacity is below 50%.

※Caution: For safety reasons, please backup all of your model data to the CF card. You can use the current model data after successfully completing the update.

1. The updating software is in a zip file. Decompress the zip file into Windows files.

2. After decompressing the zip file, you’ll find two files and one file folder. Copy these files to the CF card using an off-the-shelf CF card reader. (The memory size required is approx. 8.5 MB.)

File 1: T12mzUpdate.dat

File 2: T12mzUpdate.exe

Folder: UPDATE

3. Turn on the updating switch located beside the CF card slot.

Updating switch

Updating mode

Operating mode

4. Insert the updating CF card into the CF card slot.

5. Turn on the power switch. After approx. 10 seconds, you should see the LCD display below.

If the updating program cannot be found on the CF card, you’ll see the following LCD display.

6. Push the rotary key. The update will start automatically. The bar graph shows the installation process.

7. When the update is completed, you’ll see the LCD display shown below.

8. Turn off the power switch. When the LED is no longer lighted, turn off the updating switch.

Software version confirmation

To be sure that the updated version of software has been installed, check the information screen on the system menu.

Version indication

【New functions】

Hardware Setting

This function is for adjusting characteristics of the sticks, switches and trims.

● System menu

● Push the “Hardware Setting” button on the system menu. You’ll see the following LCD display. The hardware reverse function is the same as before.

Stick Setting

This function adjusts the stick response and histerisis (null control). It enables you to tailor the controls in each flight condition with your preferred “feel”.

- Push the “Stick Setting” button on the hardware setting screen. The LCD display below will appear.

Response setting button

Histerisis setting button

Flight condition

Group/ single selection

Adjusting the stick response

1. Push the response button for the stick you want to adjust. The rate adjustment button appears.

2. Adjust the stick response by pushing the rate button.

Default: 4

Adjusting ranges: 1-32 (the higher the number, the slower response will be).

Adjusting the histerisis

1. Push the histerisis button for the stick you want to adjust. The rate adjustment button appears.

2. Adjust the histerisis by pushing the rate button.

Default: 1

Adjusting ranges: 0-32　(the higher the number, the greater the histerisis)

Push the “Stick Setting” button after you’ve finished the setting.

Swash Mixing (Heli Mixing)

The software update adds a trim selection button.

Trim mode

● Trim on /off setting

You can select mixing characteristics either with trim or without trim.

Changes to operation after model changes and frequency changes

The current software requires that you turn off the power switch when the model is changed or frequency is changed. With this new version of software, a confirmation screen appears after you have made such changes. You will not be required to turn off the power switch but can continue operating the radio uninterrupted.