1. TT ESC Programming Features

Programming the TT ESC is as simple as answering a few questions. The TT ESC asks questions by “beeping” a setting number, followed by the possible setting values.

There are five settings that can be programmed in the TT ESC:

1) Cutoff voltage,

2) Brake Type,

3) Timing Advance and

4) Cutoff Type

5) Change the rotation of motor.

As the programmer, you must make the choice of the setting values as they are presented by the TT ESC. The setting values are “beeped” and "flashed" by the LED.

When answering a question, you will need to move the transmitter stick to the full throttle position and keep it there for about 2 seconds. After that, the TT ESC will return to the programming mode and ask the next question.

You are not required to continue through all five programming options. You can choose what item you want to change in the programming mode and the change will be retained when you leave the programming mode.

2.0 Entering Programming Mode

The TT ESC software is designed to make it difficult to accidentally enter programming mode, therefore it may seem like a long process to enter this mode. This is to prevent entering programming mode while preparing to fly or while in flight.

To enter programming mode, follow the steps below:

2.1 Verify Normal Operation

If this is the first time the TT ESC has been used, it is important to verify that the TT ESC operates normally with your transmitter otherwise programming may not function properly. Once you have verified that the TT ESC operates normally, proceed to 2.2 below.

2.2 Enter Programming Mode

2.2.1  Remove battery power from the TT ESC.

2.2.2  Move the transmitter stick to the top position (normally full “On”).

2.2.3  Reconnect battery power to the TT ESC.

2.2.4  After approximately 2 seconds, the TT ESC will emit a short tone, and the LED on, the TT ESC will give a short flash, that means power on . Then, after waiting for 5 seconds, you will hear four short “beeps” and six long “beeps”, repeated. That means it have entered programming mode.

2.2.5  As the long “beeps” occur, move your transmitter stick to the lowest position to select an option as in the table below show.

| 2.2.6 Proceed to Section 3.0 – Programming the TT ESC TT ESC “beep” | Operation | Setting | TT ESC Responses |
| --- | --- | --- | --- |
| Four short “beeps” |  |  |  |
| First long “beep” | Put stick to the lowest position | Cutoff voltage | One “beep” |
| Second long “beep” | Put stick to the lowest position | Brake Type | Two “beeps” |
| Third long “beep” | Put stick to the lowest position | Timing Advance | Three “beeps” |
| Fourth long “beep” | Put stick to the lowest position | Cutoff Type | Four “beep” |
| Fifth long “beep” | Put stick to the lowest position | Change rotation | Five “beeps” |
| Sixth long “beep” | Put stick to the lowest position | Leave programming mode | Wait 2 seconds for one or two “beeps” |

3.0. Programming the TT ESC:

Important Note: When answering a question, you will need to move the transmitter stick to the yes (full “On” throttle) position and keep it there for about 2 seconds. When the TT ESC has accepted your answer, you will get responses, then return to the programming mode and be asked the another question.

3.1 Programming Setting 1 –Cutoff Voltage

★Factory default settings are indicated by an asterisk in the option listings below.

| TT ESC “beep” | Setting | Recommended for use with: | Operation | TT ESC Responses |
| --- | --- | --- | --- | --- |
| one short “beep” |  |  |  |  |
| First long “beep” | 4.8V | 6 cell Ni-MH packs | Put stick to the top position | One “beep” |
| Second long “beep” | ★5.6V | 7 cell Ni-MH or 2 cell Li-poly packs(2.8v/Cell) | Put stick to the top position | Two “beeps” |
| Third long “beep” | 6.0V | 8 cell Ni-MH or 2 cell Li-poly packs | Put stick to the top position | Three “beeps” |
| Fourth long “beep” | 7.2V | 9 cell Ni-MH | Put stick to the top position | Four “beeps” |
| Fifth long “beep” | 8.4V | 10 cell Ni-MH or 3 cell Li-poly packs(2.8v/Cell) | Put stick to the top position | Five “beeps” |
| Sixth long “beep” | 9.0V | 12 cell Ni-MH or 3 cell Li-poly packs | Put stick to the top position | Six “beeps” |
| Seventh long “beep” | 11.2V | 4 cell Li-poly packs (per cell 2.8v) | Put stick to the top position | Seven “beeps” |
| Eighth long “beep” | 12V | 4 cell Li-poly packs | Put stick to the top position | Eight “beeps” |

3.2 Programming Setting 2 –Brake Type

| TT ESC “beep” | Setting | Recommended for use with: | Operation | TT ESC Responses |
| --- | --- | --- | --- | --- |
| two short “beeps” |  |  |  |  |
| First long “beep” | ★No brake |  | Put stick to the top position | One “beep” |
| Second long “beep” | Brake |  | Put stick to the top position | Two “beeps” |

3.3 Programming Setting 3 –Electronic timing advance

| TT ESC “beep” | Setting | Recommended for use with: | Operation | TT ESC Responses |
| --- | --- | --- | --- | --- |
| three short “beeps” |  |  |  |  |
| First long “beep” | ★4°~10° | 12 or more pole motor and out runner motor | Put stick to the top position | One “beep” |
| Second long “beep” | 10°~20° | 6 or 8 pole motor | Put stick to the top position | Two “beeps” |
| Third long “beep” | 20°~30° | 2 or 4 pole motor | Put stick to the top position | Three “beeps” |

3.4Programming Setting 4 – Cutoff Type

| TT ESC “beep” | Setting | Recommended for use with: | Operation | TT ESC Responses |
| --- | --- | --- | --- | --- |
| four short “beeps” |  |  |  |  |
| First long “beep” | Cut off |  | Put stick to the top position | One “beep” |
| Second long “beep” | ★Reduce power |  | Put stick to the top position | Two “beeps” |

3.5 Programming Setting 5 – Change the rotation of the motor

| TT ESC “beep” | Setting | Recommended for use with: | Operation | TT ESC Responses |
| --- | --- | --- | --- | --- |
| five short “beeps” |  |  |  |  |
| First long “beep” | ★ |  |  |  |
| Second long “beep” |  | Change rotation | Put stick to the top position | Two “beeps” |

3.6 Programming Setting 5 – Leave programming mode

Please see 2.2.5

Made in China  Entire Contents © Copyright 2005