1; Drill before cut the unnecessary part.. 2; Cut the unnecessary part with hacksaw or frp cutter .. 3; Add the final polish for the cutting line with sandpaper .. 4; The photo of the hull unnecessary part cutting .. 5; Sign the flexable cable cutting line . Only flexable cable length : 220 mm. 6; Cut the flexable cable with wire cutter .. 7; Do the final polish for the end of flexable cable.. 8; At first cut the 6.5 mm length brass tube for use in strut.. 9; Insert the length 6.5 mm brass tube in strut.. 10; Sign the cutting line for brass tube. Length : 195 mm..

11; Cut the brass tube with hack-saw.. 12; Mark where penetrate fixing bolt and brass tube`s hole using strut mount .. 13; Bore by drill exactly in marked position. Brass tube : 7 mm. 14; Insert the brass tube in strut mount.. 15; Bore by drill exactly in marked position. Strut mount bolt : 4 mm. 16; Picture that strut mount is attached.. 17; Assemble after have removed moter mount and servo mount on hand.. 18; Elaborate suface of mount that assemble.. 19; After assemble moter mount and servo mount, fix using instant glue.. 20; Use the instant glue evenly in whole mount for waterproof...

21; Assemble coupling mount in motor mount.. 22; Infix a brass tube in coupling mount. Inserted length : 9 mm. 23; Mark where do sandpaper, motor mount and servo mount . union area with hull to improve coherence.. 24; Grind marked area by sandpaper.. 25; Include tie for servo fixing in servo mount.. 26; Use the instant glue for temporary union motor mount and servo mount with hull.. 27; Mounts temprrary union completed picture.. 28; Prepare epoxy the 5 minutes or 30 minutes. 29; Unite part that unite temporary using epoxy.. 30; Brass tube end of transom unites using epoxy. ( Fixing of a brass tube and waterproof effect)..

31; Assemble strut .. 32; Assemble water cooler and collet to motor.. 33; Assemble motor in motor mount.. 34; Attach two face tape in the floor under mini servo that is used for rudder control.. 35; Fix durably the rudder servo in servo mount with tie that insert in mount.. 36; Bore rod hole according to position of rod for rudder. Hole size : 3 ~ 4 mm Attention : After bore, confirm whether action of rudder rod is good.. 37; Cut rubber boot. Length : 6 line. 38; Unite rubber boot using instant glue.. 39. 40; Assembly completion picture...