# RC - FM Receiver 41Mhz

# synthesised frequency

# by P. TECHER

Special thanks to Neil GILLIES for his help in this english translation.

WARNING:

The author is not responsible of loss of material, specially aircraft, due to some mistake ... this document is free of charges and don’t have any commercial value.

Please, if you improve this design by changing some parts or by improvement of the program, post me a message at:

Philippe.TECCHER@icare.fr

Remember I’m the first to be interested in, because I already built this receiver.

P.  TECHER

## Introduction:

As all R/C-modellers will tell you, you must have more than one R/C receiver - at least one for each plane! Receiver prices are high, and as a builder, I prefer to do everything myself.

This receiver allows you to receive signals sent by a standard transmitter on the French authorised band: 41.000MHz to 41.200 MHz in 10KHz steps.

In order to save money on crystals, this receiver synthesises the desired frequency.

It uses the latest technology - two mixer stages (10.7MHz and 455KHz) and microprocessor signal decoding. It is exceptionally small and weighs only 60 grms.

It can decode a maximum of eight channels and receive any analogue signal with positive or negative modulation, from two to eight channel transmitters.

Before starting this project, it is necessary to have some electronic equipment available:

A voltmeter, 100MHz oscilloscope, 50mV sensitivity / 100 MHz Frequency counter and if possible, an HF FM generator. Top of the equipment list would be an HF spectrum analyser !

You will also need a good magnifying glass, a lot of patience, tweezers, epoxy resin and some courage!

The construction and adjustment is easy to do by any competent person. Cost (in France) is no higher than a 4 channel standard commercial receiver.

## Principle:

The following block-diagram gives a good idea of the complexity of the RF stage. Fortunately, the number of  components are  reduced by using a Motorola IC.

The frequency is programmed via a small external switch module. This module is external to avoid extra volume and weight.

## FM Receiver

The input amplifier uses an FET connected in common gate mode, thus allowing gain control by adjustment of the drain current. The remaining parts of the schematic are implemented on two chips: MC3362 and MC145170.

Except for the two SMD chips, the receiver does not use any specialised components.

After the FET, the first LC filter passes only the frequency we are interested in. Impedance conversion with the first mixer is done by another LC circuit. A crystal filter is required to pass the desired frequency (BW: 7KHz at -3dB). A second 455KHZ filter - CFU 455G - gives a bandwidth of 9KHz at -6dB.

The VCO is tuned by an LC tank, to change the frequency, we just change the capacitor value of the LC tank with varicap diodes. In order to increase the voltage range for the desired band,  we increase the polarisation current on the diodes by adding two 12K resistors.

Frequency variation can be up to 4 MHz, this increases the noise sensitivity of the loop.

With an LC tank, we can easily obtain a 400KHz variation for a 0-4V input voltage.

## Synthesised frequency:

The MC145170 chip has all the elements of a standard PLL - divider, phase comparator and a 3 wire interface (HCK, DATA, CS). The dividers are programmed by the microcontroller to obtain a 1KHz common ratio.

The integrated oscillator in the MC145170 is not used here, we just capacitively tap the 3362 local oscillator output. This avoids the extra expense of another crystal!

Loop stability is very important here. It is unacceptable for an aircraft to crash due to a PLL stability problem !.

A Laplace loop approximation allows us to compute the stability factor:

with:

Kd : relation between phase and voltage (Vcc/p)

F(p): filter transfer function.

Ko : relation between frequency and VCO command voltage (100KHz/V)

Ho, the open loop transfer function:

Ho(p)=Kd.F(p).Kop.N

when the loop is closed, we have:

H(p)=Ho(p)1+Ho(p)

The loop filter is built with two RRC circuits:

Transfer function F(p) is:

F(p)=1+R2.C.p1+(R1+R2).C.p

with:

T1=R2.CT2=(R1+R2).C

and,

T1 = R2.C = 39K.1µF = 0.039

T2 = (R1+R2).C = (3,3K+39K).1µF = 0.0423

N  = 30400

Ko = 100KHz/V = 105.p

Kd = 4/p

Ho(p)=4.105324001+T1.pp.1+T2p

We can trace asymptotic Bode curves:

The phase margin is enough to have sufficient stability in the loop. The cutoff frequency is low enough to delete 1KHz and harmonics. A second RC Filter deletes all remaining HF traces in the VCO control voltage.

## Power and microcontroller:

This receiver must run with a voltage requirement < 5V and with minimum current consumption. That is the reason why we have choosen a PIC16C57 microcontroller. It programs the EEPROM, PLL and also decodes the analogue RC signal. Due to lack of information on PCM coding, I have not included PCM decoding as a feature in the software.

A 4MHz crystal synchronises the controllerÕs clock. Crystal current is limited by a 220W resistor to avoid crystal damage due to the ageing.

PIC also controls a piezo-electric transducer which is driven by a small SMD transistor. As the current consumption of a piezo transducer is very low, the transistor is here to do a hypothetical drive of an external buzzer. The transducer is a circular plate (25mm diameter). It must be spot glued around the edge to the base of the box in order to get the maximum noise.

The HF circuitry and PIC will run at 3V without any problems, the EEPROM needs slightly more - 3.5V - for programming.

A 3.3V low drop-out regulator gives a 4.1V output using a resistor divider to ground.

Power stability is extremely important for HF parts. A small voltage variation will modify the reference oscillator (10.245 MHz) which disturbs the capacitive link with the 145170. The PLL loses lock, and the receiver is disturbed.

With the regulator, the receiver is protected against PLL lock problems (and also high current drain due to servo jitter).

The PIC software allows the synthesised frequency to be programmed with simple external switches.

At power up, a single BEEP shows that all is OK, three BEEPs indicates a read defect on the EEPROM.

Frequency programming is only done at switch on by reading the switch configuration. If there is no switch, the PIC reads 0000 and no programming is performed.

If the EEPROM write was successful, it gives two BEEPs followed by a start-up BEEP.

If there is a problem during EEPROM write, (voltage too low) it gives three BEEPs.

When there is an EEPROM problem, the receiver is automatically configured to receive 41.020MHz.

If there is a lock defect with the PLL in the 100mS following switch on, the system emits a siren sound and locks the PIC program in an infinite loop until RESET.

The PIC watch dog is not used here.

The servo signal can be either positive or negative going, the software doesnÕt care. The PIC software decodes the RC signal using the following rules:

- pulse width 55µS < Tp < 385µS

- command pulse width 0.752mS < Tv < 2.7 mS

- synch. pulse width: Tsync > 4.096mS.

These rules avoid the characteristic servo jitter during signal loss.

If, after 2.5S, no valid RC signal is detected, the receiver emits four BEEPs every 2S.

The following table shows the programming codes. Codes 28 and 29 enable receiver tuning in 1Khz steps.

| Frequency | N | B4 | B3 | B2 | B1 | B0 | Remark |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NORMAL MODE | 0 | 0 | 0 | 0 | 0 | 0 |  |
| 41,000 MHz | 1 | 0 | 0 | 0 | 0 | 1 |  |
| 41,010 MHz | 2 | 0 | 0 | 0 | 1 | 0 |  |
| 41,020 MHz | 3 | 0 | 0 | 0 | 1 | 1 |  |
| 41,030 MHz | 4 | 0 | 0 | 1 | 0 | 0 |  |
| 41,040 MHz | 5 | 0 | 0 | 1 | 0 | 1 |  |
| 41,050 MHz | 6 | 0 | 0 | 1 | 1 | 0 |  |
| 41,060 MHz | 7 | 0 | 0 | 1 | 1 | 1 |  |
| 41,070 MHz | 8 | 0 | 1 | 0 | 0 | 0 |  |
| 41,080 MHz | 9 | 0 | 1 | 0 | 0 | 1 |  |
| 41,090 MHz | 10 | 0 | 1 | 0 | 1 | 0 |  |
| 41,100 MHz | 11 | 0 | 1 | 0 | 1 | 1 |  |
| 41,110 MHz | 12 | 0 | 1 | 1 | 0 | 0 |  |
| 41,120 MHz | 13 | 0 | 1 | 1 | 0 | 1 |  |
| 41,130 MHz | 14 | 0 | 1 | 1 | 1 | 0 |  |
| 41,140 MHz | 15 | 0 | 1 | 1 | 1 | 1 |  |
| 41,150 MHz | 16 | 1 | 0 | 0 | 0 | 0 |  |
| 41,160 MHz | 17 | 1 | 0 | 0 | 0 | 1 |  |
| 41,170 MHz | 18 | 1 | 0 | 0 | 1 | 0 |  |
| 41,180 MHz | 19 | 1 | 0 | 0 | 1 | 1 |  |
| 41,190 MHz | 20 | 1 | 0 | 1 | 0 | 0 |  |
| 41,200 MHz | 21 | 1 | 0 | 1 | 0 | 1 |  |
| 40,990 MHz | 22 | 1 | 0 | 1 | 1 | 0 |  |
| 41,210 MHz | 23 | 1 | 0 | 1 | 1 | 1 |  |
|  | 24 | 1 | 1 | 0 | 0 | 0 |  |
|  | 25 | 1 | 1 | 0 | 0 | 1 |  |
|  | 26 | 1 | 1 | 0 | 1 | 0 |  |
|  | 27 | 1 | 1 | 0 | 1 | 1 |  |
| Adj - 1 KHz | 28 | 1 | 1 | 1 | 0 | 0 |  |
| Adj +1 KHz | 29 | 1 | 1 | 1 | 0 | 1 |  |
|  | 30 | 1 | 1 | 1 | 1 | 0 |  |
|  | 31 | 1 | 1 | 1 | 1 | 1 |  |

The PIC software includes a soft signal filter. The software is required to do time sampling. This sampling - at 10µS - does not allow a stable servo output. We can observe a 1step oscillation around the servo home position.

To avoid this, we do the following:

- if the receiving pulse is less than 1 step of the last, we put the last on the output and memorise the new width.

We have a filter which does not suppress the pulse dynamics.

## The box:

This is a great problem ! It must be as light as possible, strong and to the required dimensions of the PCB.

It is built with 0.6mm plywood and epoxy resin.

Glue the parts together with cyano (except the cover). Inside, lay in 125g/M glass fibre with epoxy resin. After 24 hours, cut the base with a knife.

Finish by gluing the top and putting again some resin inside. At the same time, make a small tongue (with resin, balsa and scotch tape - to avoid gluing) and put it on the base.

Glue the 2mm nut with resin. make a hole for the tongue and small holes for the buzzer and configuration switches.

To finish, sand the box with sandpaper and fill with car primer. Another fine sanding and two coat of paint will give you a professional mirror finish.

## Assembly and adjustment:

The HF parts are on a small PCB  (33 X 35mm). SMD devices give some extra room. Begin by solder vias, chips, resistors and capacitors then finish with the HF transformer. Ensure that you assemble the components in this order as there is very little space between components. The crystals should be mounted using silicon RTV adhesive.

After soldering all the components, clean the PCB with alcohol and a toothbrush to eliminate any traces of flux.

The programming connector is built with some IC tulip pins. Solder these to the PCB and lock in place using a loop of tinned copper  wire.

Build the switch module using  VERO-BOARD (strip board). Solder a DIL switch with 1K resistors connected to Vcc . The left pin is Vcc, with the two pins on the right for the buzzer.

Connect a piezo buzzer to the controller PCB and ground ÇÊPLL input lockÊÈ by a resistor. Power up the controller board only, the buzzer must give three BEEPs followed by a siren sound (lock defect).

Verify that the voltage level after the regulator is correct about 4,1V. Program 41.100 MHz on the switch module and power up the board. The module will then emit two BEEPs just before the siren noise - the  EEPROM is now programmed.

Place solder pins at the resistor on  the VCO input, at the 3362 audio output (just before zero crossing detector)  and at the divider VCO input of the PLL.

Connect the HF board with 3cm (max) of wire to the controller board. Switch on, the current consumption must be below 20mA.

Verify with a scope and a frequency-meter that the Fref PLL output gives 10.245 MHz, adjust if necessary using a  capacitor.

Connect the frequency-meter to the VCO PLL input and an adjustable voltage to the VCO command input. Adjust the LC tank to obtain 400KHz variation for 3V variation of the VCO control.

Power up a transmitter (or HF generator) and adjust tyhe 455KHz transformer to obtain a good signal on the 3362 audio output. Put the transmitter far enough away to see the AGC running, the lower the received signal, the more the drain voltage increases. When the drain voltage is half of supply voltage, adjust the antenna transformer to decrease this voltage to zero, in the same way, adjust the transformer at the 3362 input.

The adjustments are now complete, you can do the real test - near the ground -. Put the transmitter in a car and connect a servo to the receiver. Get a commercial receiver on the same frequency and do the same thing.

Get a friend to drive the car 400 meters (1200ft) away, verify that the servo still functions. Continue to drive the car further away, continually verifying that the response of the synthesised receiver is as good as the commercial receiver.

Do not forget forget to verify the receiver selectivity. Use two other transmitters on adjacent  frequencies to see if there are no intermodulation problems.

Remark:

The PIC software will handle any transmitter with two to eight channels.

## Parts List:

| Quantity | Reference | Value |  |  |
| --- | --- | --- | --- | --- |
|  | CPU part |  |  |  |
| 2 | C1,C2 | 18pF |  |  |
| 3 | C3,C4,C6 | 10µF |  |  |
| 2 | C5,C7 | 100nF |  |  |
| 1 | C8 | 1µF |  |  |
| 1 | Qz | 4Mhz (SMD) |  |  |
| 1 | Q1 | 2N2222 (SOT23) |  |  |
| 8 | R1,R2,R3,R4,R5,R6,R7,R8 | 390 |  |  |
| 2 | R9,R11 | 220 |  |  |
| 1 | R10 | 10K |  |  |
| 1 | R12 | 1K |  |  |
| 1 | R20 | 4K7 |  |  |
| 5 | R21,R22,R23,R24,R25 | 47K |  |  |
| 1 | U1 | PIC16C57 |  |  |
| 1 | U2 | 93C46 (SMD) |  |  |
| 1 | U3 | LP2950CZ-3,3 (TO92) |  |  |
|  | HF parts |  |  |  |
| 1 | Cv | 33pF variable (SMD) |  |  |
| 2 | C1,C3 | 18pF |  |  |
| 2 | C2,C6 | 100pF |  |  |
| 6 | C4,C8,C9,C10,C12,C18 | 100nF |  |  |
| 1 | C5 | 10µF |  |  |
| 1 | C7 | 22pF |  |  |
| 2 | C11,C14 | 220pF |  |  |
| 1 | C13 | 1µF |  |  |
| 1 | C15 | 330pF |  |  |
| 1 | C16 | 47pF |  |  |
| 1 | C17 | 22nF |  |  |
| 1 | F1 | CFU455E ou G |  |  |
| 1 | F2 | crystal 10,7MHz filter |  |  |
| 3 | L1,L2,L3 | TOKO 2K509 |  |  |
| 1 | L4 | TOKO 4102 black |  |  |
| 1 | Qz1 | 10,245 MHz |  |  |
| 1 | Q1 | J310 |  |  |
| 2 | Q2,Q3 | BC859C (SOT23) |  |  |
| 1 | R1 | 1K |  |  |
| 1 | R2 | 100 |  |  |
| 1 | R3 | 100K |  |  |
| 1 | R4 | 2K7 |  |  |
| 1 | R5 | 33K |  |  |
| 1 | R6 | 2K2 |  |  |
| 1 | R7 | 33K |  |  |
| 1 | R8 | 2K2 |  |  |
| 2 | R9,R11 | 3K3 |  |  |
| 1 | R10 | 10M |  |  |
| 1 | R12 | 150K |  |  |
| 3 | R13,R14,R15 | 12K |  |  |
| 1 | U1 | MC3362 (SMD) |  |  |
| 1 | U2 | MC145170 (SMD) |  |  |