- Original post: http://simplewords.genexis.net/?page_id=642

- SK360 Flybarless eRaptor

Introduction

With the recent advent in flybarless technology in RC helicopters, there are increasing number of products that offers this feature at an increasingly affordable price. Some of them comes with a 3-axis gyro, and others only comes with 2. To list a few:

- SK360 (http://www.skookumrobotics.com/)

- V-Stabi  - VBAR (http://www.vstabi.de)

- Gyrobot

- Acro Control - 3X

- Helicommand

- CSM

For this flybarless project, i’ll be using the SK360 system.

Reasons are because:

- It is a 2-axis system, which means I can still use my current gyro, which is a Futaba GY611 w/ 9256, for my tail control. This leaves me with one axis less to setup on the flybarless system as I am already very comfortable with setting up my tail with the GY611 gyro system.

- Affordability and Availability. Among all other flybarless systems, SK360 is the most easily available to me in terms of shipping and purchasing. It only costs US$16 to ship it from Canada to Singapore, and it arrived within 6 days! I received the shipping order less than 24 hours from the time of payment. Compared to the rest of the systems in the market, the SK360 is by far the most affordable of them all, costing US$329.

- Online Support. SK360 is well supported by the users online. There are at least two threads on them, with users discussing about setup and posting pictures of their conversions. This two threads can be found on Helifreak and Runryder. Click on the links to access them. On top of that, the makers of SK360 has been constantly fixing issues and updating firmware in the past months. This can be seen from the updates on the right side of their website.

- Simplicity. I really like the integration of everything into one small rectangular box. It makes the system compact and easy to mount.

- Finless. Everybody knows finless. When finless says its good… damn it is good. Finless has a video of the setup and discuss thread on the SK360 flybarless system right here. It is really resassuring to know that finless is also using this flybarless systems as if all else fails, finless is there to support you

I’ll be using one of my eRaptors for this flybarless project. The setup is as follows:

- Thundertiger E550 (upgraded to 600 blades and boom)

- Custom Lower A123 compatiable frames (see them here!)

- Castle creations HV85 (SetRPM Governor Mode)

- Neumotors H1912/1.5Y

- Futaba GY611 w/ S9256

- Hitec HS-6985HB for cyclic servos (0.16s, 10+kg)

- JR 2.4Ghz technology for TX and RX

For more information regarding my eRaptor builds, do visit this build log.

The SK360 came with the following:

- SK360 2-axis Gyro unit

- USB adapter to connect the gyro unit to the USB cable

- USB extension cable to link the PC to the USB adapter

- 3 x 3-pin extension wire to connect the Gyro unit to your RX

- PC Software (version 1.03)

- User manual (version 1.05)

Swash and Servo Setup

After trying to follow the manual to setup the helicopter, i’ve decided to rearrange some of the steps to make the whole process easier. Also, i’ll high light on some steps which are mention in the manual, but the users may miss it. Please keep in mind that my setup is for a eCCPM 90degree swash. However, it should not defer too much if you are using a 120degree swash or something else.

Radio setup

The first thing you should do, is to change your swash type to a NON-eCCPM (mCCPM) type, just like what you would do for when you setup a Raptor 30/50/60/90 helicopter. In my X9303/DSX9, it is the NORM swash type (1servo). You NEED to do this even if you are using a CCPM helicopter such as the align trex series or TT E550/E620 (the one i am using).

Next, reset all the end points adjustments, sub trims, reverse for the cyclic servos. You should no longer have the swash mix menu since you already selected the swash type to be a non-eccpm one. Again, you need to do this even if your servos do not line up 90 degrees with your swash. All these trims will be done in the PC software itself later, so do not worry.

Lastly, make sure you have at least one mode where your pitch curve varies from 0-50-100% (preferably linear).

Software installation and PC connection

While setting up the radio, you can also take this time to install the PC software that came with the SK360. If you have misplaced it, or you would like to get the latest version (mine came with the latest), do visit http://www.skookumrobotics.com/ for the latest download.

When it is completed, connect up the SK360 unit to your RX. DO NOT connect the servos yet. Next, connect the SK360 unit to your PC via the USB adapter, remember to make sure that the SK360 unit is in SETUP mode. Start up the PC Software. Turn ON your TX, and your RX. Check the PC Software to ensure that it is showing CONNECTED (green) at the top left hand corner of the program.

After the above steps, the SK360 is now connected to your PC. Click on the Swashplate Tab and set the Swash type for your helicopter. In my case, i am using the eCCPM 90degree swash setting. Also ensure that the “Mixing Location” is set to “in Gyro”.

Next, click on the Servos Tab. Input the parameters from for your cyclic servos. For my setup, i am using digital servos. Here is the important step. Connect your servos to the SK360 unit as indicated on the user manual. Now, you are going to trim your servos such that it lines up 90 degree with your swash. Do the same as what you would do when you are setting the helicopter up with your radio. For me, i just eye ball the servo horns to make sure that it is almost straight. I also ensured that my TX pitch input is at the 50% position, which means zero pitch in idleup1. To help me out, i used a swash leveler (slight modded to suit 90 and 140 degree swash) to ensure that when at zero pitch, my swash is leveled

Once the swash setup is done, it is time to install the main rotor head.

Main Rotor Conversion

I believe in every flybarless conversion, the most interesting part is the main rotor head conversion. It is a test of creativity and functionality. In my conversion, i’ve decided to make use of my current rotor head, bought from www.xoluxionhobby.com. One of the crucial part of the conversion is making the swash follower. The swash follower is used to ensure that the top rotating part of the swash plate moves with the main shaft during flights. There are the more expensive methods, such as purchasing from mikado. Or the cheaper methods such as modifying the current washout base and washout arms to perform the same function.

In my conversion, I will be modifying my washout base and washout arms which were originally on my eRaptor. I had a crash and as a result, one of the washour arms was bent. Hence, i’ll be using it for the conversion. I used dremel and shaved off the original washout arm (top), until i get some like this (bottom).

I drilled a 2.5mm hole, and threaded a 3mm screw thread so that i can use 3mm set screws to hold the washout base to the main shaft. In the picture below, the left hole is the original hole used to attach the washout arm, and the right hole is the one created by me for the set screws.

When we put everthing together, this is what we get. Genexis’s Swash Follower.

The next step is to install the main rotor hub. After removing the flybar accessories, I needed to extend the length of the main rotor grip linkage such that it will line up with the swash. For my conversion, i used 3mm(id) 6mm(od) washers to line them up. Total extended length by the washers is 11mm.

The length linkage rod for my setup is around 66mm.

It really helps when you have the proper tools.

And we put everything together, this is what we get As for the phase ring, it is adjusted such that when the linkage balls of the swash lines up, the linkage ball of the main rotor grip is also lined up with it.

Top view

Mid stick, zero pitch.

Lowest stick, max negative pitch. This is before the swash mixing setup.

Highest stick, max positive pitch. This is before the swash mixing setup.

If this is done correctly, there should be minimum phasing problem. You can test if you have phasing issues by following the instructions stated in the SK360 manual, pages 7 and 8. I will ensure that the pitch is zero at mid stick. But to do that, i’ll need to attach the blades to the rotor grips, which is a little troublesome but not crucial for now, hence it will be done in later parts of the setup.

Software Setup

This part, in my opinion, is probably the most boring but important part of the flybarless setup. By now, the swash. main rotor head should be completed. So the next in line is the PC software. The instructions in the SK360 manual is pretty clear in explaining how the unit should be configured. Take special note of pages 11 and 12, read every sentence and follow accordingly.

Just to high light a few parts:

- Under the “Inputs from Receiver”, the values should be close to zero. If they are not, you can offset them or trim them in the Control tab->Inputs from receiver (different from above)->Trim(%). Play around and you will understand its function easily.

- In the latest firmware, there is a “Show Actual Gains” button where you can see the actual gains used by the SK360 unit in flight. The reason for such a function is because there are two ways you can adjust the bell and hiller gain, one is at the PC software, the other is at the gain dials located at the sides of the SK360 unit. Hence, the values showed in the screen when this button is pushed will be the gains used by the SK360 unit. For a start, i followed the instructions in the manual, and set the Bell gain to 30%, and Hiller gain to 50%.

- All the other gains were left as default. I’ll update agian after i perform a test flight.

This is a very brief discussion for the PC software setup. Please read the instruction manual or wait for finless’s setup videos for the detailed walk through.

Pitch Settings

Now that we have something almost functional, the next step is adjusting the pitch. It is recommended by the makers of SK360 to have at least 10degrees of cyclic pitch. Please note the difference between cyclic and collective pitch. Cyclic pitch is the amount of pitch the blades will have when you input aileron or elevator commands when the collective pitch is at zero, meaning that the pitch control stick is at the centre.

To increase or decrease the pitch, we will use the “Swash Mixing” in the “Swash plate” Tab in the PC Software. One of the biggest problem with flybarless setups is that there is nothing we can use to take reference to when we use the pitch guage. I came around this problem by inserting a stick (in this case a R50 landing skid) into the top of my rotor head, and took it as the reference horizon.

Looks good enough for a zero pitch

When the collective pitch is zero, the pitch on the blades should also be zero. If this is not zero, do not make any changes to the sub trims of the servos. Instead, adjust the length of the linkage rods which connects the main rotor grips to the swash. Once you mange to get zero pitch at mid sticks, you can adjust your maximum and minimum pitch using the swash mixes. For my setup, I made it such that i attain a +/- 13 degrees pitch, with a swash mix of 35%.

Do the same for aileron and elevator such that you will get a pitch of +/-10degrees. For instructions of how the blades should be positioned when setting up the aileron and elevator pitches, refer to pages 7 and 8 of the SK360 manual.

Pictures of the linkages after swash mixing has been completed.

Mid stick (zero degrees)

Max pitch (13 degrees)

Min pitch (-13 degrees)

Wiring

Once those are done, the last step to this setup is to mount the gyro unit and arrange the wires. It is important that the gyro unit is mounted on a flat and smooth surface for maximum mounting strength . The front of the gyro unit MUST be facing the absolute front of the helicopter, if not there will be mixing issues in flight. Take note of page 4 of the SK360 manual.

There isn’t much to talk about in this section, hence i’ll just post the pictures of my wiring and arrangement of the gyro unit.

Top view

Left view

Right view

Conclusion

In conclusion, conversion to flybarless is not as intimidating as others may perceive it. The SK360 unit is pretty easy to setup, and has a lot of features and parameters which you can tweak to improve flight characteristics. As of now, I have no done any test flights, but i’ll post updates and videos when they are done.

Somethings which i dislike about the SK360:

- The whole unit is made of plastic and feels a little cheap. None the less, the functions and the price makes up for this.

- There is a lack of on site setup tool such as a LCD control panel like what the VBAR has. However, this is not really needed because the only parameters that needs to be tweaked on site are the bell and hiller gain. These two parameters can be adjusted via the gain dial at the side of the gyro unit. However, more advance parameters such as the damping gain and others requires a laptop to configure on site. Thus, i hope that in the near future such a tool will be made available to the users of SK360.

- I do not like the input and output pins to be facing upwards. It makes mounting difficult as i have to bend and rearrange the wires to face the side (See picture 1 in previous page). I hope that in future release, all outputs and input ports of the SK360 unit will be at the side, like the JR R921 receivers.

- Thats about all

Flybarless conversion completed !

Once i have finalized the parameters and setup of this flybarless conversion, i will wonder milling off the top part of my main rotor hub to look something like this: