- http://66.196.80.202/babelfish/translate_url_content?.intl=us&lp=ja_en&trurl=http%3a%2f%2fwww.oyajin.jp%2f%7etoko%2fpic%2f0032%2findex.html

- Infrared ray transceiver NO.12 (PIC12C509A receiver)

[Previous] [Next] [Index]

Channel 3 where it is possible to use servo, infrared ray receivers

It tried making the infrared ray receiver which can replace to the RF receiver that way. If there is an infrared ray receiver which can use servo the utilization range becomes wide.

But the functional part passed 3, because of sense power source the tip/chip tantalum capacitor of 33μF was added one for smoothly. The PIC very thing operates even below 5V, but because rating of the infrared ray light absorbent element is 5V, in the lithium electric battery 1 cell hindrance arises in operation. Therefore at voltage above the lithium ion 2 cell, it means to use in the form which supplies power source from BEC of the speed controller.

The usual way it is possible to assemble with aerial wiring, but this time it tried using the both sides glass epoxy baseplate of 0.5mm. Because mounting the part is completed in one side, it peeled the copper foil of one side which is not used. Furthermore because it is simple pattern, this time using the cutter knife, it cut pattern. As for baseplate size 11mmx16mm.

The infrared ray light absorbent element and between the substance made the length of 400mm with the copper wire of 0.1mm as 3 these twists. As for weight in state of picture 1.1g (receiver 0.9g + IR sencer 0.2g). While flying being essential for the light absorbent element if possible to install to the position where from the transmitter it does not become the shade if necessity it can use the plural light absorbent elements.

Because the latest receiver just has distributed serial pulse of input every channel in real time, when radio disturbance occurs, it means that way to be reflected on servo and the speed controller. The person who uses those which have fail-safe is safe in the speed controller. In order to prevent radio disturbance, as much as possible it is necessary to set to the position where the light absorbent element from the transmitter does not become the shade. You can use the good point by the transmitter of the manufacturer of any. In addition, also it is possible to connect the magnetic actuator driver in place of servo.

| 3 CHANNELS IR RECEIVER IRXS300 | 3ch-dec1.hex for 12C509A |
| --- | --- |