# YS-X6飞控与地面接收软件接口协议

20120514

Custom data transmission format

Transmission characteristics: baud rate 115200 8 bits no parity 1 stop bit no hardware flow controlData Format: without stated are the least significant bit firstDownlink:Output ports: On the completion of the power-on initialization, flight control COM1 and COM3 are so agreed output.Transmission order of the beginning of the frame flag, data, parity bit, finished a wait 200ms repeat renewal, or 5 frames per second.

共99字节：

- the flag at the beginning of frame：（0~3）

$STP

- GPS data：

Aircraft Latitude				4 bytes		float,单位为度，IEEE浮点格式，低位在前，下同（4~7）

Aircraft longitude			4 bytes		float,单位为度  （8~11）

Target point Longitude		4 bytes		float,单位为度  （12~15）

Target point latitude	    4 bytes		float,单位为度  （16~19）

The heading angle			4 bytes		float，radians（20~23）due north to 0, clockwise is positive

Star Search		1 byte		uchar,将其直接显示即可，单位为颗  (24)

Year				1 byte		uchar                              （25）

Month			1 byte		uchar                              （26）

Day				1 byte		uchar                              （27）

When			1 byte		uchar                              （28）

Points			1 byte		uchar                              （29）

Seconds			1 byte		uchar                              （30）

Uploaded total number of waypoints	1 byte       uchar，(31)，only for automatic navigation

- manual rudder format：

Rudder 1 byte UCHAR, 100 to 200, unsigned integer, 0 for no input (32)

Aileron 1 byte UCHAR, 100 to 200, unsigned integer, 0 for no input (33)

Elevator 1 byte UCHAR, 100 to 200, unsigned integer, 0 for no input (34)

Throttle 1 byte UCHAR, 100 to 200, unsigned integer, 0 for no input (35)

- actual rudder surface state：

Rudder 1 byte uchar, 100 ~ 200, unsigned integer (36)

Aileron 1 byte uchar, 100 ~ 200, unsigned integer (37)

Elevator 1 byte uchar, 100 ~ 200, unsigned integer (38)

Throttle 1 byte uchar, 100 ~ 200, unsigned integer (39)

- Other：

_Y To speed 2 bytes ushort filtering unit for cm / s (40 to 41)

Boot time 2 bytes ushort (42 to 43), seconds

Reserved 1 byte char (44)

Reserved 1 byte uchar, (45)

Distance from the take-off point (high) byte UCHAR, (46), in meters * 256;

PTZ around point radius of 1 byte char (47), in meters

Pressure from 2 bytes int height, the unit decimeter (48 to 49)

GPS velx 2 bytes ushort, unit cm / s (50 to 51)

Distance from the take-off point (low) byte UCHAR, (52), in meters;

Receiver status byte char (53) 0 is open, also receive remote control commands, off, automatic state

Shake coefficient 1 byte uchar (54)

PDOP value of 1 byte uchar, (55)

Vibration coefficient 1 byte uchar (56)

Temperature 1 byte uchar, in degrees, (57)

Right to the acceleration 2 bytes short (58 to 59)

Backward acceleration 2 bytes short (60 to 61)

Pitch angle 4 bytes int (62 to 65) in degrees, Yang is overlooking the negative

Roll angle 4 bytes int (66 to 69) units for the degree of left-right negative

Flight control voltage value 2 bytes ushort (70 to 71) the value / 4096 * 25 voltage value

The downward acceleration of 2 bytes short (72 to 73)

Current task number (to which point) 1 byte uchar, from 0 to 255 unsigned integer (74), from 0 to 1 point

Current control status byte UCHAR, 0x00 manual, 0x01 auto-hover automatic navigation, 0x02, 0x04 to the point, 0x07 to set state, 0x08 gyro cleared 0x0b return landing (75)

Power consumption 2bytes ushort, (76 ~ 77), in mA

Alarm flag 1 byte UCHAR (78) is 1, low voltage, the phone should vibrate, is 0, no alarm;Filtered _D the 1 byte uchar,, velocity (high) units of cm / s * 256 (79)

The rudder position 1 byte UCHAR, (80)Aileron-bit byte UCHAR, (81)

The flipper bit 1 byte uchar (82)

After filtering _X 1 byte uchar speed (high), in units of cm / s * 256 (83)

Set target height 2 bytes, int, in decimeter, (84 ~ 85)

Reserved 3 bytes ushort (86 to 88)

Filtered _D 1 byte uchar, speed (low) to cm / s (89)

Reserved 3 bytes ushort (90 to 92)

Filtered _X 1 byte uchar, speed (low) to cm / s (93)

GPS vely 2 bytes ushort, unit cm / s (94 to 95)

Version No. 2 byte (96 to 97)

- End of the frame checksum：

Cumulative number of bytes from the header to the previous bytes (98 bytes) unsigned single。uchar (98)