U2 flight control instructions (version 2.0)

November 22, 2013

update U2 flight control OSD display interface data definitions:

OSD数据解释：

Location Azimuth aircraft: an aircraft relative to the takeoff site, now located, such as:  Refers to the aircraft is now in place in relation to fly south direction (0 degrees north, 180 degrees south).

Aircraft forward: refers to the direction of the aircraft now forward, such as: ， Refers to the direction of the aircraft is 53 degrees (0 degrees north) forward.

Aircraft return angle: the deviation angle of the aircraft at the time to go home, as  Refers to the aircraft 53 degrees to the left, the direction of home.。Another example: ，Refers to the aircraft 68 degrees to the right now, as the direction of home! Return angle is one of the most important data! With particular attention.

Linear distance: refers to the position of the aircraft on the ground away from home to a linear distance in meters.

Height: relative to the "home" position relative to height. The unit is meters.

Voyage: Aircraft cumulative flight distance.

T: Flying throttle size, this value is set in the lowest throttle and consistent, easy to judge when the throttle flight

RSSI: This is Flight Control RSSI voltage wiring port, the unit is 0.1V, measuring voltage range of 0-3.3V. Display R32, said voltage is 3.2V. This port is used to connect segments of strong receiving airport, to show good and bad receiver signal, a receiver 433 if it is, it shows that the packet loss rate, voltage 3.2 (or 3.3) for receiving a signal of 100%, 0 is out of control. According to this value, we can see the status of the receiver.

Aircraft attitude: Display the aircraft pitch angle and tilt angle, according to a small airplane icon, you can see in real time the aircraft attitude. As

Representing aircraft rightist 3 degrees, the aircraft nose down 1 degree.

Off timer: When the throttle more than 3 0%, 1 second or more, the timer starts counting. When the start time, set the mode to get in, you must re-connect power before the timer does not start counting, to enter the setup mode.

Beijing Time: refers to flight time, can be used for recording.

Radar: intermediate representation "home" position, said the boats wing aircraft, flying wing direction indicates the direction of the aircraft forward. On the radar screen, the north-east left west right, said the aircraft made ​​at the location.

Angle illustrating:

OSD settings interface definition:

Flight control board wiring definition:

Marked "G" on the board is the ground!

Figure pass the camera after the power input inductor-capacitor filter, the output of two image transmission camera power output. So the camera image transmission power input voltage input how many volts, image transmission power output on the camera how much volt power output, after the LC filter inside. So inputs needed to connect to the appropriate power supply (general image transmission and camera voltage of 12V), 3S, they can directly power electric and image transmission camera input directly connected (with a small switch icon on the board 2 positions) achieve total electricity use. Under the above voltage 4S case, add their own buck circuit, connect the power down to the rear camera image transmission power input.

Figure pass and camera power connection wiring diagram:

1: 3S battery when connected to a total power used by two points on the map :( will directly shorting cap can be shorted, after an internal LC filter to the camera and image transmission 12V supply)

2: External independent power supply diagram:

3: 4S-6S battery power, the external power supply schematic buck:

Current meter and GPS wiring diagram:

Connection galvanometer attention! Be sure to connect after a good line, check there is no problem, connect the battery, do not live flight control plug wires! Remember Remember!

Current sensor wiring diagram:

Input signal:

1: Aileron channel

2: The horizontal tail channel

3: Throttle channel

4: Mode Select Channel

(Before GPS is not available, the three sections are all balanced mode switch control, only received six stars above, it can be converted to the other control mode.

Three segments of the flight mode switch can be achieved between the three types of flight mode switch:

1: Balance Mode: three upscale segment switch position.

Flight Control is responsible for maintaining the aircraft's attitude relative to the ground. Pine pole achieve level flight aircraft.

2: Lock mode (version 2.0 or later): three sections of the switch in the middle position.

Lock mode is the aileron channel or pitch channel, when the rocker back to the mid-point, the flight control is automatically locked direction or height mode, more than the midpoint, then converted to balanced mode. Such a lock mode, you can achieve given the high directional flight, scheduled non-directional high-flying, high-flying directional uncertain balance mode four kinds of flight mode, very easy to use.

3: return mode: three low-end segment switch position.

Automatic return flight control, throttle decided by the lowest throttle and throttle receiver, go where large, the actual throttle output. If the receiver is set to return runaway protection mode, you can achieve out of control, the aircraft automatically return.

5: Manual control channel

(switching full manual mode and flight control modes unless the flight master, it is strongly recommended that empty without connecting to abandon manual mode!)

Output signal:

1: Aileron channel (no aircraft aileron servos connected to the vertical tail channel)

2: The horizontal tail channel

3: throttle channel (ESC connection)

4: 4 output channels, connecting one side of the V-tail aircraft tail, such as "fat boy" the normal type of aircraft do not use

If you need to connect the general layout of the aircraft rudder, rudder servo is plugged directly to a 4-channelreceiver position.

Points for attention:

One, FPV aircraft ESC 5V supply higher (heavier aircraft, steering and other heavy load)! You must use UBEC 5V power down, the nominal 3A above, linear power current, fever, it is not conducive to safe flight. Strong hands, it is recommended between the flight control or 5V and ground receivers in parallel with a capacitance 3300UF about steering interference.

Two: Figure pass camera power Note: Figure pass and camera power supply is connected, generally require 12V voltage (if it is 5V image transmission, power positive can be connected directly to the 5V power transfer output), so only the 3S case under electric power and image transmission can be a total power usage (power supply can be directly output and image transmission power input short-circuit cameras connected to the internal LC filter for the camera and image transmission power). Under the above circumstances 4S, image transmission and camera power input location, must be connected to a regulated 12V power supply (or 12V independent power supply).

Three: return of the two aircraft out of control one correct premise failsafe settings. Second, set the time of the "home" position . Two conditions are indispensable, so pay attention.

In addition to cut cake 433 receiver failsafe other receivers are not permanent and may be lost at any time (such as the first open receiver, after opening control, you will lose a lot of receiver failsafe!). If you use the receiver general, each time after takeoff, you need to shut down the control test is correct return; set home position, the number of suggestions and then eight or more satellites, the GPS data so that it is stable, before take-off, check the straight-line distance from the house not more than 10 meters, in order to confirm the position is to set a good home, and then to fly away! Especially novices, FPV be careful! Every time to do all the checks, then voyage.

Flight control adjustment steps:

First, in accordance with the input and output channels connected to the remote receiver: connect the monitor to see OSD settings interface, 1-8 rows correspond to the settings as follows.

(OSD settings interface can appear premise is that the timer does not start counting! When the throttle channel of more than 3 0% 1 second or more, start the timer, then there would not be setting interface, you need to set up, you must re-connect electricity.)

1. the pros and cons of the remote control to adjust the aileron and pitch channel to channel:

The throttle to the lowest channel, turn the battery, breaking move right aileron channel rocker (magnitude should be enough), hold for 5 seconds, if the debugger interface appears, then reverse the aileron channel settings are correct, if there is no OSD Set interface, then the aileron channel is inverted. Breaking up the right aileron channel, OSD settings interface can occur.

OSD settings interface appears, bobbing pitch channel up and down the pitch joystick remote control, if the display up and down arrows and action consistent, positive and negative pitch channel is set correctly. Otherwise, reverse pitch channel.

2. set the parameters

Positive and negative feedback phase and adjust the size of roll control.

The arrow points to the AIL channel feedback coefficient, pitching up and down rocker, the data will be 120 -120 ---- +, positive and negative feedback, said direction, choose the right direction and magnitude of the feedback (usually plus or minus 80-1 between 00, if the feedback is too big, it will cause jitter)

Positive and negative feedback phase and adjust the size of the pitch control.

The arrows point to the feedback coefficient ELE channel up and down the pitch rocker, data will be 120 -120 ---- +, positive and negative feedback, said direction, choose the right direction and magnitude of the feedback (usually plus or minus 80-100 between, if the feedback is too big, it will cause jitter)

Set return mode, the lowest throttle ratio.

Move to THR-L settings interface, up and down the pitch rocker refers to data changes between 10-100. Under the indicated return mode, the minimum is 30% to 100% throttle.The return value is the lowest throttle mode (in other modes, the user complete control throttle), if the actual throttle is greater than the minimum throttle, the actual throttle output (for example: 60% minimum throttle setting, 70% of the actual output of the receiver, the flight control according to the 70% output, set the minimum throttle 60%, the actual throttle to 10%, then 60% of the output of the accelerator. Under the return mode, the lowest throttle trigger condition for over 4 kilometers per hour, if the return mode, the moving speed of more than 4 kilometers per hour, the minimum throttle automatically trigger otherwise in accordance with the actual size of the output of the receiver throttle benefits of this design are: throttle failsafe if the receiver is zero throttle, after the plane landed, because they do not move, the throttle is automatically closed.

。

Set return height.

Move to set the height of the interface return, the joystick up and down the pitch, the value will turn 50, 100, 150 ... ..500 changes in between --- represents most of the total height of the return of ---, 50 meters, 100 meters . . . . . (---) That keep cutting height (return process, the height unchanged) when returning, the return process, the autopilot will follow a certain percentage, gradually reducing or increasing height. Until it finally reaches the set level. Be sure to give enough attention to the throttle, if not lower than the throttle to maintain level flight, the aircraft is prone to spiral. Throttle size according to their own aircraft, the own hands.。

Set Aircraft type:

There are two kinds of aircraft types to choose from, flying wing mode and normal mode. Choice, the right aileron channel breaking move, until the aircraft type to the corresponding icon.

Set Home Location:

If the satellite GPS positioning over 6, set the arrow placed on the set home position, the right aileron path breaking move, hold for 5 seconds, and tests show GPS "wait ..." said that it has set up successfully. The same place, just need to set the time, after the same location, it can be retained until the man-made changes. This setting is very important to ensure that the aircraft is an important condition for the safe return home.

。

Sensor calibration:

Each time a new installation to a plane, to be calibrated once. The aircraft to maintain the real level flight attitude (wings remain around the level, under the wing of the wing surface elevation around 5 - 8 degrees), remain absolutely still does not move, then move to the right aileron breaking channel, hold for 5 seconds until " hourglass "icon, remain motionless, waiting for sensor calibration, time of about 10 seconds to complete the calibration (during the aircraft remains stationary fixed).

Voltage calibration settings:

Breaking down the pitch move the joystick will adjust the voltage, this voltage value is adjusted to the correct voltage value.

Exit and save: breaking move to the left aileron channel until you exit the setup interface is saved. All saved data is permanent, unless the man-made changes.

。

Flight Control Flight mode graphical explanation:

Given the high mode: constant height to keep the aircraft flight control, can be controlled directly in accordance with the left and right balance mode. It should be noted at this time, the throttle enough to not less than the minimum throttle level flight. Otherwise it will cause the aircraft to spiral.

Given the high directional mode: aircraft flight control to maintain constant altitude and heading. This mode is suitable to fly away, so do not adjust the direction of the Personnel Division, is operating in the most practical FPV)

Directional indefinite High mode: keep the aircraft flight control heading angle, height into balanced mode, so you can climb or reduce the height of the courier.

Automatic return mode: according to the set height (or maintain a high level), the automatic return, it is noted that in this mode. Throttle is to choose the lowest setting the throttle and the receiver output in one of the largest accelerator, as the output of the throttle control ESC.。

Balanced Mode: The lever control aircraft flight control posture. Let go, the plane automatically reply to level flight.

Manual mode: Full manual flight, flight control not to participate in the control (in this mode requires a certain level of operation, it is strongly recommended not to use in FPV can not connect the input channel 5, the default completely abandon manual!.)

Frequently Asked Questions:

1: Aircraft at high speed or dive time, about why the plane would shake?

A: The aircraft aileron feedback coefficient bit big, big feedback value will cause the aircraft to emerge about shock, with due emphasis on the value of small, aircraft will be stable, between positive and negative recommendation generally set 70-90.

2: flight control damping measures need not need to?

A: The need to give due consideration to the flight control damping, particularly oil machine or near the motor mounting distance flight control aircraft. Many damping approach. Milton vacant, foam blocks, coins and other flight control pad. General electric aircraft, only moderate treatment can, in most cases ,, for flight control of vibration insensitive to little effect.

3: U2 flight control and actual coordinates show how very different?

A: There are a variety of formats to display latitude and longitude, this is in accordance with the flight control display GPS standard output format, degrees, divided into60 binary; second is percent if converted into 60 hex when data needs to multiply after the decimal point 60.

Specific conversion is as follows:

OSD Display: E10428.344 - Longitude 104 degrees, 28 minutes, 20.64 seconds (60 * 0.344 = 20.64)

OSD Display: N4347.789   - Latitude 43 degrees, 47 minutes, 47.34 seconds (60 * 0.789 = 47.34)

4: Flight Control in balanced mode switch to another mode, the aircraft will suddenly decrease (or climb) meters, and then to stabilize.

A: There are three possible, an aircraft center of gravity is wrong, two servo neutral point is no good tune, three flight control is no calibration.

The solution, as long as either one aspect of a good adjustment, you can. Proposed to adjust the order: In manual mode, the remote control to fine-tune the set, and adjust the steering gear, the aircraft can fly steady posture. Then follow the instructions, calibration flight control. You can eliminate this phenomenon.